Title: Voice Recognition for Robot Control

Project Type: Project in AIS

Assigend Student(s):  Mithila Maruti Prabhu -mithila.prabhu@stud.fra-uas.de
Romana Rashid – romana.rashid@stud.fra-uas.de

Group No – VRC-7

Start and End Date: 01/Dec/2025 – 20/Mar/2026

Research Problem/ Hypothesis/ Motivation (Why do I want to do what I will do?)

Voice control is a natural interface for human-robot interaction (HRI), offering hands-free operation ideal for applications like telepresence, assistive guidance, or simulated search and rescue. However, deploying reliable voice recognition for direct robot control, even in simulation, faces significant practical barriers:

- Pronunciation and Dialect Variability: Off-the-shelf speech recognition models perform poorly with non-standard accents or non-native speech, creating an accessibility gap in globally usable systems [1].

- Noisy Speech in Simulated Environments: While the simulation itself is quiet, the user's environment (e.g., a busy lab) is not. Real-world background noise drastically increases Word Error Rates (WER), leading to incorrect navigation commands like "rotate left" being misheard as "move left" [2].

- Inflexible and Limited Command Sets: Most robot voice control demos use a small, hard-coded command list (e.g., "forward", "stop"). This fails to accommodate natural language variation ("go ahead", "please turn right", "move 0.5 meters") and cannot scale to new tasks without code changes.

- Simulation-Specific Latency: The pipeline from microphone input in the real world to observable action in the Gazebo simulator must be fast enough to feel responsive and maintain user trust.

Therefore, the core hypothesis is that a voice recognition system for the TurtleBot, designed with accent robustness, noise resilience, and a dynamically scalable command database, will achieve higher reliability, flexibility, and user satisfaction in simulation-based testing compared to standard ASR APIs.

Goal/ Objective (What do I want to achieve?)

This project aims to develop, integrate, and evaluate a robust Voice Recognition Control (VRC) system for a simulated TurtleBot3 within the ROS/Gazebo framework, directly addressing the challenges of pronunciation, noise, and command flexibility.

Specific Objectives:

- Implement a Robust VRC ROS Node: Develop a ROS package (turtlebot_vrc) containing:

  - An audio processing module capable of real-time capture and basic enhancement.

  - A fine-tuned Speech Recognition engine, optimized for a defined robotic command lexicon and varied accents.

  - A scalable Natural Language Understanding (NLU) module that uses a structured, editable database (e.g., YAML/JSON) to map diverse natural language utterances to precise ROS Twist messages or action goals.

- Achieve Seamless Gazebo Integration: The node will subscribe to audio topics or services and publish velocity commands to control a simulated TurtleBot3 (e.g., burger or waffle model) in a standard Gazebo world (e.g., turtlebot3_world).

- Quantitative & Qualitative Validation: Systematically evaluate the system's performance by measuring:

  - Command Success Rate (%) across three critical test scenarios: clear speech, accented speech, and speech with ambient noise.

  - System Latency (ms): From utterance start to observe robot movement in Gazebo.

  - Vocabulary Expansion Success: Ability to correctly handle new commands (e.g., "dock", "circle the obstacle") added to the database post-deployment.

Methodology (How do I want to achieve it?)

Phase 1: Research & System Design (Weeks 1-4)

- Literature & Tool Review: Survey ROS audio_capture/publish packages, open-source ASR models suitable for fine-tuning (Whisper, Vosk), and NLU tools (Rasa, simple intent parsers).

- Architecture Design: Create a detailed ROS node graph. Define the data flow: Microphone → [ROS Audio Topic] → VRC Node (ASR → NLU) → /cmd_vel → TurtleBot in Gazebo.

- Command Schema Definition: Design the structure for the scalable command database, specifying intents (navigation, manipulation), sample utterances, and corresponding ROS message parameters.

Phase 2: Development & Integration (Weeks 5-11)

- Simulation Environment Setup: Install and configure ROS (Noetic/Humble), TurtleBot3 simulation packages, and Gazebo.

- VRC Node Implementation:

  - Develop the audio interface using roslibpy (for web-based mic access) or a native audio_capture node.

  - Integrate and fine-tune a selected ASR model using a custom dataset of navigation commands recorded in varied conditions.

  - Build the NLU module that loads the command database and performs keyword/spelling-corrected matching.

  - Ensure the node outputs standardized geometry_msgs/Twist messages.

- Testing in Simulation: Conduct incremental tests in Gazebo, starting with basic movement commands.

Phase 3: Experimental Validation (Weeks 12-14)

- Design Test Protocols:

  - U1 (Baseline Accuracy): 50 standard commands in a quiet room.

  - U2 (Accent Robustness): Same commands spoken by 2-3 individuals with distinct accents.

  - U3 (Noise Robustness): Commands given with played-back cafe/office noise.

  - U4 (Scalability Test): Introduce 5 new complex commands (e.g., "go to the red wall") into the database and test recognition.

- Data Collection & Analysis: Automate test execution where possible, logging all metrics. Compare results against a baseline system (e.g., unmodified Whisper model).

Phase 4: Documentation & Reporting (Weeks 15-16)

Deliverables (How will I deliver my results?)

- Final Report: Comprehensive documentation of the project.

- Functional System Artifacts:

  - A ROS package (turtlebot_vrc) hosted on a Git repository, ready for deployment.

  - A clear architectural diagram (ROS node graph).

  - A results table summarizing key metrics (Success Rate, Latency) across all use cases U1-U4.

- Conclusion Summary: Example: "The implemented VRC system, using a fine-tuned Whisper-small model and a YAML-based intent database, achieved a 94% success rate for standard commands (U1) and maintained over 85% in noisy conditions (U3). It successfully interpreted 4 out of 5 newly added commands (U4) without code changes. Primary failure mode remained extreme accent deviation combined with noise."

- User Guide: A concise README.md detailing setup, installation, and how to add new voice commands to control the TurtleBot in simulation.

References (if already available)

[1] A.B. Baker, …., In: IEEE EUROCON 2025 (Alternatives: My observations in ASIS lab, or: my observations in elderly care house, or: Professor N’s observations, or: …..)

[2] N.D. Maier, …., Individual Project Report, FRA-UAS 2025

[3] …….

[4] …….

Remark: Apply the KICS (Keep It Clear and Structured) principle. Use facts and metrics as well as clearly formulated, specific, logically ordered, and consistent statements.